SPECIFICATION

Customer: QILONG

| Item: | Crystal Unit |  | Receipt |
| --- | --- | --- | --- |
| Type: | NX3225SA |  |  |
| Nominal Frequency: | 26 MHz |  |  |
| Customer’s Spec. No.: |  |  |  |
| NDK Spec. No.: | EXS00A-CS10135 |  |  |

| Revision Record |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rev. | Date | Items | Contents | Approved | Checked | Drawn |
| --- | 23.Aug.2016 | Issue | --- | M.Sato | --- | R.Omomo |
| A | 14.Oct.2016 | 4. Electrical Specifications | Change: Frequency tolerance | M.Sato | --- | R.Omomo |

1. Customer’s Spec. No.                                    :

2. NDK Spec. No.                                               : EXS00A-CS10135

3. Type                                                                : NX3225SA

4. Electrical Specifications

|  | Parameters | SYM. | Electrical Spec. |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min | typ | max | Units |  |
| 1 | Nominal frequency | fnom | 26 |  |  | MHz |  |
| 2 | Overtone order | - | Fundamental |  |  | - |  |
| 3 | Frequency tolerance | - | -10 | - | +10 | ×10-6 | at +25C |
| 4 | Frequency versus temperature characteristics | - | -50 | - | +50 | ×10-6 | at -40~+125C The reference temperature shall be +25°C |
| 5 | Equivalent resistance | - | - | - | 100 | Ω | IEC -Network Series |
| 6 | Load capacitance | CL | - | 8 | - | pF | IEC -Network |
| 7 | Level of drive | - | - | 10 | 200 | µW |  |
| 8 | Insulation resistance | - | 500 | - | - | MΩ | When terminal to terminal and terminal to cover were applied at DC100V ±15V. |
| 9 | Operating temperature range | Topr | -40 | - | +125 | °C |  |
| 10 | Storage temperature range | Tstr | -40 | - | +125 | °C |  |
| 11 | Air-tightness | - | - | - | 1.1×10-9 | Pa m3/s | Helium leak detector |

5. Examination results document

Since a performance is guaranteed, an examination results document does not submit.

6. Application drawing

6.1 External dimension			: EXD14B-00370

6.2 Taping and reel figure			: EXK17B-00098

6.3 Holder marking			: EXH11B-00317

6.4 Reliability assurance Item		: EXS30B-00499

6.5 Recommendation reflow profile	: EXS30B-00344

7. Notice

7.1 Order items are manufactured according to specification. As to conditions, which are not indicated in this specification and unpredictable such as applied condition and oscillation margin, please check them beforehand.

7.2 Unless we receive request for modification within 3 weeks from the issue date of this NDK specification sheet, we will supply products according to this specification. Also, if you’d like to modify specification of order, which has been placed with delivery request within 3 weeks from the issue data of this specification sheet, we would like to discuss with you separately.

7.3 In no event shall the company be liable for any product failure resulting from an inappropriate handling or operation of the product beyond the scope of its guarantee.

7.4 Where any change to the process condition is made due to the change(s) in the production line, inform personnel of the specifications.

7.5 Should this specification data give rise to any disputes relating to any intellectual property rights or any other rights of a third person, the company shall not indemnify anyone for any damage. Their disclosure must not be construed as the grant of a license to use any of the intellectual property rights owned by the company.

7.6 If you intend to use products listed on this specification for applications that may result in loss of life or assets (controls relating to safety, medical equipment, aeronautical equipment, space equipment, etc.), please do not fail to advise us of your intention beforehand.

7.7 In the company’s production process whatever amount of ozone depleting substances (ODS) as specified in the Montreal protocol is not used.

7.8 Information contained in this specification must not be quoted, reproduced or used for other purposes including processing either in part or in full without obtaining prior approval from the company.

7.9 Crystal units will be damaged by ultrasonic welding process due to resonance of crystal　wafer itself. NDK does not recommend using ultrasonic welding. If Ultra Sonic welding used, NDK strongly recommend verifying crystal unit damage by ultrasonic weld.

7.10 The appearance color and so on have a different case by purchasing it more than 2 suppliers　of the component, but characteristic and reliability are guaranteed.

7.11 In case of the product long time keep at high temperature and humidity, may affect product characteristic (solder ability) and a packing condition.

Please keep at storage condition of temperature +5C ~+35C, humidity ~85%RH.

8. Prohibited items

Be sure to use the product under the following conditions. Otherwise, the characteristics deterioration or destruction of the product may result.

(1) Reflow soldering heat resistance

Peak temperature: 265C, 10 sec

Heating: 230C or higher, 40 sec

Preheating: 150C to 180C, 120 sec

Reflow passage times: twice

(2) Manual soldering heat resistance

Pressing a soldering iron of 400C on the terminal electrode for four seconds (twice).

Terminal

Tungsten

Metalize

（

Au

Plating

on Ni pre-plating)

Base

Alumina

Ceramics

Cover

Metal

(1)

1.2

(1)

#1

#3

#4

#2

#2

#4

#3

#1

PIN CONNECTION

（

TOP

VIEW

)

※

#1,#3 : Xtal

#2,#4 : GND (CONNECTION COVER)

2.2

1.4

LAND PATTERN (TYPICAL)

C0.3

3.2±0.1

2.5±0.1

0.55±0.05

(0.9)

(0.9)

0.7

1.2

1.6

Reliability assurance item

(page: 1/1)

| No. | Test Item | Test Methods | Specification Code |
| --- | --- | --- | --- |
| 1 | High Temperature Storage | +125±3C 1000h | A,D |
| 2 | Low Temperature Storage | -40±3C 1000h | A,D |
| 3 | Temperature Humidity | +85±3C 80~85%RH 1000h | A,D |
| 4 | Temperature Cycling | -55±5C / +125±5C It is 1000 cycles using 30 minutes each as 1 cycle. | A,D |
| 5 | Vibration | Frequency Range : 10~2000Hz Amplitude or Acceleration : 1.52mm or 196m/s2 1 cycle : 20 minutes Test time : Three mutually perpendicular axes each 4 hours. | B,D |
| 6 | Shock | Devices are shocked to half sine wave (49000m/s2, 0.15msec) six mutually perpendicular axis each 1 times. | B,D |
| 7 | Drop | Devices are dropped from the height 75cm onto iron plate. Execution 3 times random drops. | B,D |
| 8 | Solderability | Pre-heat temperature : +150±10C Pre-heat time : 60~120s When the temperature of the specimen is reached at +215±3C, it shall be left for 30±1sec. Material: H63A (Silver 23%) Flux : Rosin resin methyl alcohol solvent ( 1 : 4 ) | C |
| 9 | Reflow resistance | Pre-heat temperature : +150~180C Pre-heat time : 90±30s Heat temperature : more than +230C Pre-heat time : less than 30s Peak temperature : +260±5C Peak time : less than 10s | B,D |

| Specification code | Specification |
| --- | --- |
| A | f/f £  20 ppm CI/CI £  15 % or 5  make use larger value |
| B | f/f £  10 ppm CI/CI £  15 % or 5  make use larger value |
| C | The electrodes should be covered by a new solder at least 90% of immersed area. |
| D | After testing unless cracking of materials view of eyes and unless break of seal. |

Recommendation reflow condition

1.IR  reflow  condition